MARYLAND OCCUPATIONAL SAFETY AND HEALTH

SAFE OPERATION OF AN AERIAL LIFT

An aerial lift is any vehicle-mounted device used to elevate personnel, including extendable boom platforms, articulating (jointed) boom platforms, vertical towers, and any combination of above. Many workers are injured or killed on aerial lifts each year. The major causes of fatalities are falls, electrocutions, and collapses or tip overs. This Bulletin provides safety tips on operating aerial lifts.

- Ensure that workers who operate aerial lifts are properly trained in the safe use of the equipment.

- Maintain and operate elevating work platforms in accordance with the manufacturer's instructions.

- Never override hydraulic, mechanical, or electrical safety devices.

- Never move the equipment with workers in an elevated platform unless this is permitted by the manufacturer.

- Do not allow workers to position themselves between overhead hazards, such as joists and beams, and the rails of the basket. Movement of the lift could crush the worker(s).

- Maintain a minimum clearance of at least 10 feet, or 3 meters, away from the nearest overhead lines.

- Always treat power lines, wires and other conductors as energized, even if they are down or appear to be insulated.

- Use a body harness or restraining belt with a lanyard attached to the boom or basket to prevent the worker(s) from being ejected or pulled from the basket.

- Set the brakes, and use wheel chocks when on an incline.

- Use outriggers, if provided.

- Do not exceed the load limits of the equipment.

- Allow for the combined weight of the worker, tools, and materials.

For Assistance Contact Us

CONSULTATION SERVICES call (410) 527–4472 for workplace surveys, hazard evaluations and suggestions on hazard elimination.

MOSH Outreach						 (410) 527-2091

REGIONAL OFFICES:

Region 1 (Eastern Shore Counties) 				(410) 819-5860

Region 2 (All Maryland Counties)				(410) 527-4442

Region 3 (Baltimore City) 					(410) 527-4426

Region 4 (Garrett, Allegany, Washington, Frederick &		(301) 791-4699

Montgomery Counties)

Region 5 (Carroll, Howard, and Anne Arundel Counties) 		(410) 527-4458

Region 6 (Prince George’s, Calvert, Charles and St. Mary’s Counties)	(301) 883-3317

Region 7 (Baltimore County, Harford, and Cecil Counties)		(410) 527-4434

We are here to help you